SwitchLinc 2-Way Programmer

name:	cmd_SwitchLinc.txt

version:	1.0

author:	Glenn Steiner

Macro allows a user to easily and quickly setup SwitchLinc 2-Way switches.  All 10 remotely programmable functions are implemented.  Programming is best done via an X10 Maxi-Controller.  User feedback and steps are spoken by the Microsoft Agent.

When macro is run  user only needs to press a couple of buttons to program a function.  P1 through P16 are used to issue programming commands.   Commands:

P1 ON:	 Remotely Set Ramp Rate

P2 ON:	 Remotely Set On-Level

P5 ON:	 Set Scene Membership

P6 ON:	 Remove Scene Membership

P7 ON:	 Set Scene Ramp Rate

P9 ON:	 Disable X-10 Transmit

P10 ON:	 Re-Enable X-10 Transmit

P11 ON:	 Disable Programming

P12 ON:	 Enable Programming

'P16 Off:	Exit Setup

How to use example:

Set House Code on Maxi-Controller to P

Start the macro cmd_SwitchLinc

To set Scene Membership of one or more switches:

Press 6 and then ON on the controller

You will be instructed to:

Set Scene Members and On-Level: Set Switches On-Level and Press On

Adjust switch(es) as desired and press ON on the controller.  You will be instructed to:

Transmit Scene Address for Membership

Do so by:

Setting desired Scene House Code on the Maxi-Controller.

Pressing the Unit Number desired on the controller.

The light(s) will flash.  You are done.

Remember to reset the House Code on the Maxi-Controller to P.